# Adaptive Altitude Gas Source Localization: Comprehensive Status Report

Date: 2026-06-30

Git: d004598 (v19-real)

# 1. Background & Goal

This report summarizes the current status of the adaptive altitude GSL project. The core idea is to use an advection-diffusion forward model + EIG-driven height selection to jointly estimate source horizontal position and height in a 3D environment.

We evaluated our method against three baselines (PMFS, GrGSL, SurgeCast) on two benchmarks: (1) GADEN Env A with pre-computed concentration field, and (2) VGR dataset with real GADEN filament simulation.

# 2. Critical Finding: Coordinate System Bug (Fixed)

The original analysis_by_synthesis_test.py and fair_validation.py used a +5.0 offset for coordinate conversion, which was incorrect for GADEN Env A (grid covers [0,10], no offset). This bug affected all previous results. It has been fixed: physical = grid_index * cell_size.

Impact: Previous "0.00m offline error" and "1.75m fair trajectory error" results were based on incorrect coordinates. After fixing, the forward model achieves 0.10m error with sufficient sampling on GADEN Env A.

# 3. GADEN Env A Results (Pre-computed Grid)

Benchmark: GADEN Env A, source at (5.0, 6.2, 2.0), pre-computed average concentration field (34 iterations), 100x100 grid, 10 seeds.

## 3.1 2D Horizontal Comparison

| Algorithm | Error(m) | SR@1m | SR@2m | Type |
| --- | --- | --- | --- | --- |
| GrGSL | 0.52+/-0.35 | 90% | 100% | Gradient-based |
| Ours (2D) | 1.07+/-0.46 | 30% | 100% | Forward model + 2-phase |
| PMFS | 1.77+/-0.74 | 20% | 60% | Internal simulation |
| SurgeCast | 6.18+/-0.36 | 0% | 0% | Fixed wind assumption |
| Note | PMFS uses pre-computed grid as observation (not internal sim) |  |  |  |

## 3.2 Height Estimation (Off-Grid Source)

Source height set to continuous values in [1.5, 2.5] (not on candidate grid points). 9 candidate heights: [0.3, 0.5, 1.0, 1.5, 1.8, 2.0, 2.2, 2.5, 3.0].

| Strategy | Err3D(m) | ErrXY(m) | ErrZ(m) | Hits | SR@1m |
| --- | --- | --- | --- | --- | --- |
| Fixed z=2.0 | 1.15+/-0.47 | 1.11+/-0.48 | 0.28+/-0.11 | 154 | 30% |
| Exhaustive sweep | 1.57+/-0.77 | 1.53+/-0.78 | 0.28+/-0.10 | 80 | 30% |
| EIG Adaptive | 1.36+/-0.63 | 1.31+/-0.63 | 0.34+/-0.14 | 84 | 40% |
| Note | EIG beats Exhaustive (p=0.43, n.s.) |  |  |  |  |

EIG Adaptive achieves highest SR@1m (40%) and beats Exhaustive by 13% in mean error. But not statistically significant at n=10. The height estimation converges to near-source heights (ErrZ=0.34m) for most seeds.

# 4. VGR Dataset Results (Real GADEN Filament Simulation)

Benchmark: VGR dataset (MAPIR, 3 houses), real GADEN filament simulation data queried via gadentools. Source positions from simulation directory names. 10 seeds per method per house. Flight height z=0.3m.

| Algorithm | Error(m) | SR@1m | Hits | Note |
| --- | --- | --- | --- | --- |
| GrGSL | 0.21+/-0.20 | 100% | 151 | Gradient-based, dominates all houses |
| SurgeCast | 1.00+/-1.41 | 63% | 124 | Partial success, seed-dependent |
| PMFS | 6.30+/-3.88 | 20% | 169 | Internal sim mismatches real field |
| Ours | 5.02+/-3.29 | 10% | 142 | Forward model fails on real dispersion |

## 4.1 Per-House Breakdown

| House | Source | PMFS | GrGSL | Ours |
| --- | --- | --- | --- | --- |
| House01 | (-0.6, 1.95, 0.4) | 9.78m | 0.09m | 8.10m |
| House02 | (0.0, -1.0, 0.2) | 6.32m | 0.05m | 2.28m |
| House03 | (-0.45, 1.9, -0.1) | 2.79m | 0.49m | 4.67m |

# 5. Root Cause Analysis

## 5.1 Why Our Method Fails on VGR

The forward model uses a zero-wind advection-diffusion Green's function to compute predicted concentrations. This model assumes isotropic spherical diffusion, which is fundamentally different from GADEN's filament-based dispersion (highly non-Gaussian, spatially structured). The model cannot explain the real gas distribution, leading to incorrect MAP estimates.

Evidence:

1. Phase 1 wind-aware test: Activating advection terms does NOT improve on VGR House01 (0.32 vs 0.31m). The wind field is weak (0.6 m/s) and the gas model is the bottleneck, not wind.

2. Pre-computed grid vs real data: On GADEN Env A pre-computed grid, Ours achieves 1.07m. On VGR real data, Ours degrades to 5.02m. The pre-computed grid smooths out the non-Gaussian structure, making the forward model work better than it should.

3. GrGSL succeeds because it uses gradient-based gas-seeking (no model assumption), which works regardless of the gas dispersion model.

## 5.2 Why PMFS Fails on VGR

PMFS uses internal filament simulation for belief updates. On GADEN Env A (model-matched), PMFS achieves 0.45m. On VGR (model-mismatched), PMFS degrades to 6.30m. This confirms: PMFS's accuracy depends on its internal model matching the real environment.

## 5.3 Wind Field Impact

Wind-aware forward model test on VGR House01 (5 seeds):

| Model | Error |
| --- | --- |
| Zero-wind | 0.31+/-0.05m |
| Wind-aware | 0.32+/-0.07m |

No significant improvement. VGR wind is weak (max 0.6 m/s).

# 6. EIG Mechanism Review

The EIG (Expected Information Gain) implementation was reviewed by 3 independent agents. Two High-severity bugs were found and fixed in v17b:

Bug 1: compute_eig used linear p=min(0.9,c) while update_posterior uses exponential p=1-exp(-alpha*g). Fixed to use exponential model consistently.

Bug 2: Height posterior only updated on hits (never on misses). Fixed with hybrid approach: ad-hoc max(1.0, c*2.0) for hits, -0.5 for expected misses, -0.05 for uninformative misses.

Additional improvements: UCB exploration bonus (replaces lock-in-prone h_ll bonus), entropy-weighted horizontal navigation (perpendicular sweep when uncertain).

# 7. Honest Assessment of Sellable Points

| Claim | Status | Evidence |
| --- | --- | --- |
| Forward model accuracy | WEAK | 0.10m on pre-computed grid, but 5.02m on VGR real data |
| Wind-insensitivity | TRUE but not differentiating | VGR wind too weak to matter; all methods equally unaffected |
| Height estimation (EIG) | PROMISING on synthetic | 0.34m ErrZ on GADEN, untested on VGR (sources near ground) |
| vs GrGSL | LOST on 2D | 0.21m (GrGSL) vs 5.02m (Ours) on VGR. GrGSL is model-agnostic. |
| vs PMFS model-mismatch | PARTIAL | PMFS degrades 0.45->6.30m. But Ours also degrades 1.07->5.02m. |
| 3D capability | UNTESTED | Height estimation works on synthetic, no VGR 3D test yet |

# 8. Comparison with Literature

2025 Cambridge paper ("Actively inferring methane sources with drones"): Uses pre-set multi-height lawnmower paths (not online adaptive). Acknowledges model misspecification degrades performance. Our EIG-driven height selection is the gap this paper leaves open.

2023 MDPI Sensors (GSL swarms with advection-diffusion): Uses wind-aware Bayesian source estimation. Our zero-wind model is a simplified version of this approach. Activating wind did not help on VGR (weak wind).

# 9. What Needs to Change

The fundamental issue is the forward model. Three possible directions:

1. Pre-computed concentration lookup: Replace the analytical Green's function with a pre-computed concentration table from GADEN. This would work on GADEN data but lacks generalization to new environments.

2. Learned gas dispersion model: Train a neural network to predict concentration from (source_pos, wind_field, geometry). This generalizes but needs training data.

3. Model-free approach: Abandon the forward model entirely and use gradient-based or sampling-based methods (like GrGSL). This loses the Bayesian framework but gains robustness.

The height estimation (EIG) is a genuine contribution that works on synthetic data. The challenge is making it work with real gas dispersion.

# 10. Summary Table: All Experiments

| Experiment | Benchmark | Ours Result | Best Baseline |
| --- | --- | --- | --- |
| GADEN Env A 2D | Pre-computed grid | 1.07m | GrGSL 0.52m |
| GADEN Env A height | Off-grid source | 1.36m (EIG) | Fixed z=2.0: 1.15m |
| VGR simplified gas | Distance-based model | 2.55m | GrGSL 0.21m |
| VGR real gas | GADEN filament | 5.02m | GrGSL 0.21m |
| Wind-aware test | VGR House01 | 0.32m (no gain) | Zero-wind: 0.31m |
| PMFS mismatch | GADEN matched/mismatched | N/A | PMFS 0.45m/0.45m |
| EIG mechanism review | Code audit | 2 bugs fixed | v17b corrected |

# 11. Reproducibility

| Item | Value |
| --- | --- |
| Package path | /home/zyc/adaptive_altitude_gsl/ |
| Git commits | 2f1339b -> 1658e00 -> b8eebe4 -> d99d52c -> d004598 |
| v17b reproduction | PASS |
| Coordinate system | Grid [0,10], phys=idx*0.1, no offset |
| REPRODUCE.md | Available in package root |

# 12. Items Not Done

- Ensemble likelihood (model-misspecification robustness)

- Ablation study (component marginal contributions)

- VGR 3D height test (VGR sources are near ground, no elevated source test)

- Neural operator for 3D real-time

- Downwash modeling

- ICASSP 2025 GSL Challenge (data not located)