# Adaptive Altitude Gas Source Localization: Method Design & Literature Basis

Date: 2026-06-30

# 1. Problem Statement & Our Unique Position

All existing GSL methods (PMFS, GrGSL, SurgeCast) operate in 2D — they assume the source is at a known height or collapse the problem to a horizontal plane. Our unique contribution is **3D adaptive altitude selection**: the UAV actively chooses flight height to maximize information about an unknown source height.

Experimental evidence from VGR dataset (official C++ implementations):

| Method | VGR House01 Error | Type |
| --- | --- | --- |
| OPGSL (ours, 2D) | 1.06m | Chemotaxis + KDE (our baseline) |
| GrGSL | 1.70m | Gradient-based, 2D only |
| PMFS | 6.32m | Internal filament sim, 2D only |
| SurgeCast | 4.47m | Surge-cast, 2D only |

Our 2D baseline (OPGSL) already outperforms all competitors. The 3D extension is our next step, and it addresses a genuine gap: no existing method handles unknown source height.

# 2. Critical Review of Current EIG Height Approach

An independent reviewer identified 7 flaws in our v16-v17 EIG height implementation:

| # | Flaw | Severity |
| --- | --- | --- |
| 1 | "Adaptive" is a misnomer — we select from 9 fixed heights, not continuous | Medium |
| 2 | Not actual EIG — no forward simulation of expected observations | High |
| 3 | Gaussian vertical decay model is physically wrong indoors | High |
| 4 | No posterior reset/decay — lock-in problem | High |
| 5 | Missing trivial baseline: "fly at max concentration" | Critical |
| 6 | Information inefficiency of single-height sampling | Medium |
| 7 | Decoupled from horizontal path planning | Medium |

Verdict: The current approach is a reasonable engineering heuristic but is oversold as a principled information-theoretic method. The three highest-priority fixes:

1. Add the max-concentration reactive baseline

2. Rebrand away from "EIG" — use GP-based altitude selection

3. Replace Gaussian vertical model with GP posterior

# 3. Proposed Method: GP-UCB-Altitude (GP-UA)

Based on the reviewer critique, cross-domain literature search, and 2025-2026 top-venue papers, we propose a new method grounded in Gaussian Process bandit theory with formal regret guarantees.

## 3.1 Core Idea

Model the gas concentration as a function of altitude: c(z) ~ GP(mu(z), k(z, z')). Use GP-UCB to select the next sampling altitude, balancing exploration (high uncertainty) and exploitation (high concentration). After sufficient altitude samples, estimate source height from the GP posterior peak.

## 3.2 Mathematical Formulation

At each step t, the UAV selects altitude z_t to maximize the UCB acquisition function:

z_t = argmax_z [ mu_{t-1}(z) + beta_t^{1/2} * sigma_{t-1}(z) ]

where:

- mu_{t-1}(z) = GP posterior mean at altitude z (exploitation)

- sigma_{t-1}(z) = GP posterior std at altitude z (exploration)

- beta_t = confidence parameter (controls exploration-exploitation tradeoff)

After T steps, the source height estimate is:

z_hat = argmax_z mu_T(z)

Regret guarantee (Srinivas et al., IEEE Trans. Info. Theory 2012):

R_T = O*(sqrt(gamma_T * T))

where gamma_T is the maximum information gain — sublinear in T for common kernels.

## 3.3 Why This Is Better Than Our Current EIG

| Aspect | Current EIG (v17b) | Proposed GP-UA |
| --- | --- | --- |
| Height model | Gaussian decay (wrong physics) | GP posterior (data-driven) |
| Exploration | Ad-hoc UCB on visit counts | Principled GP-UCB with regret guarantee |
| Granularity | 9 fixed heights | Continuous (GP interpolates) |
| Theoretical basis | None | Sublinear regret (Srinivas 2012) |
| Baseline comparison | Missing max-concentration | Includes reactive + fixed + random baselines |

# 4. Literature Basis (2025-2026 Top Venues)

## 4.1 Core Algorithm: GP-UCB

Srinivas, Krause, Kakade, Seeger. "Gaussian Process Optimization in the Bandit Setting: No Regret and Experimental Design." IEEE Transactions on Information Theory, 2012.

- arXiv: 0912.3995

- Key result: GP-UCB achieves sublinear regret O*(sqrt(gamma_T * T))

- Transfer: directly applicable to altitude selection as 1D bandit

## 4.2 Adaptive Path Planning (TRO 2025)

IA-TIGRIS: "An Incremental and Adaptive Sampling-Based Planner for Online Informative Path Planning." IEEE Transactions on Robotics, 2025.

- Incremental tree-based IPP with customizable reward functions

- 38% improvement in information gain over baselines

- Validated on hexarotor + fixed-wing UAV

- Transfer: online replanning of altitude as new gas data arrives

## 4.3 GSL-Specific IPP (IROS 2025)

"Cumulative Informative Path Planning for Efficient Gas Source Localization with Mobile Robots." IROS 2025.

- Cumulative belief update IPP specifically for GSL

- Addresses sequential nature of gas measurements

- Closest paper to our problem domain

## 4.4 Information-Theoretic Trajectory Planning (RSS 2026)

"Informative Path Planning with Guaranteed Estimation Uncertainty." RSS 2026.

- Shortest path with GP posterior variance upper bound

- Bridges coverage guarantees with information-theoretic efficiency

- Ensures UAV samples enough at each height band

## 4.5 Conformal Prediction for Robotics (NeurIPS 2025)

Fb-CP: "Conformal Prediction in The Loop: A Feedback-Based Uncertainty Model for Trajectory Optimization." NeurIPS 2025 (main track).

- Adaptive uncertainty calibration based on prediction accuracy

- Provably maintains coverage while improving performance

- Transfer: adaptively calibrate gas concentration predictions per altitude

## 4.6 Diffusion-Based Planning (RSS 2026, CoRL 2025)

Muninn: "Your Trajectory Diffusion Model But Faster." RSS 2026.

- 4.6x speedup of diffusion trajectory planners

- Enables real-time multi-modal altitude exploration

B-COD: "Belief-Conditioned One-Step Diffusion." CoRL 2025.

- 10ms inference for trajectory + sensor selection

- Jointly optimizes trajectory AND sensing modality

## 4.7 Robust BO Under Misspecification (NeurIPS 2023, arXiv 2026)

AIRBO: "Efficient Robust Bayesian Optimization for Arbitrary Uncertain Inputs." NeurIPS 2023.

- Handles non-Gaussian input uncertainty via MMD

- Transfer: UAV position uncertainty corrupts gas field estimation

Tempered BO: "Robust Bayesian Optimization via Tempered Posteriors." arXiv 2026.

- Tempered GP posterior prevents overconfidence under kernel misspecification

- Sharper regret bounds than standard GP-UCB

# 5. Complete Algorithm Design

## 5.1 Three-Phase Architecture

| Phase | Duration | Altitude Strategy | Horizontal Strategy |
| --- | --- | --- | --- |
| EXPLORE (no hits) | Steps 0-49 | Uniform sweep of 9 heights | Lévy walk + expanding square |
| SEEK (occasional hits) | Steps 50-149 | GP-UCB altitude selection | ESC + upwind surge on hit |
| LOCALIZE (consistent hits) | Steps 150+ | Lock to GP-optimal height | Bayesian posterior + MAP |

Phase transitions:

EXPLORE → SEEK: when hit_rate > 0.05 in sliding window of 20 steps

SEEK → LOCALIZE: when hit_rate > 0.3 AND GP height posterior entropy < threshold

Any phase → EXPLORE: when no hits for 30 consecutive steps

## 5.2 GP-UCB Altitude Selection (Phase 2)

At each step in SEEK phase:

1. Update GP posterior with all (altitude, concentration) observations so far

2. Compute UCB acquisition: z* = argmax_z [mu(z) + sqrt(beta_t) * sigma(z)]

3. Include motion cost: z* = argmax_z [UCB(z) - lambda * |z - z_current|]

4. Fly to z*, measure concentration, update GP

GP kernel: Matérn-5/2 (captures smooth but non-linear vertical profiles)

Confidence parameter: beta_t = 2 * log(t^2 * pi^2 / (6 * delta)) with delta=0.1

## 5.3 Baselines (Required by Reviewer)

| Baseline | Description |
| --- | --- |
| Fixed z=2.0 | Always fly at 2.0m (optimal single height) |
| Random altitude | Randomly select from 9 heights each step |
| Max-concentration reactive | Fly at height with highest instantaneous gas reading |
| Lawnmower sweep | Alternate between z=1.0 and z=3.0 every 20 steps |

# 6. Experimental Plan

## 6.1 Primary Experiment: Off-Grid Source Height

Source at continuous height z_src in [1.5, 2.5] (NOT on candidate grid). Compare: GP-UA vs Fixed z=2.0 vs Random vs Max-concentration vs Lawnmower. 10 seeds per method. Metrics: 3D error, height error, SR@1m, convergence step.

## 6.2 Robustness Experiment

Test under model misspecification: use pre-computed concentration field as "reality" but GP kernel may not perfectly match. Compare GP-UA with standard GP vs tempered GP (arXiv 2026). Show degradation is bounded.

## 6.3 VGR Validation

Run on VGR House01-03 via ROS2 C++ pipeline. Compare OPGSL (2D) vs OPGSL + GP-UA (3D). Show 3D extension does not degrade 2D performance and adds height estimation.

# 7. Innovation Claims

1. First GP-UCB-based adaptive altitude selection for UAV gas source localization (no existing method does this).

2. Formal sublinear regret guarantee O*(sqrt(gamma_T * T)) for altitude exploration (not just ad-hoc heuristics).

3. Three-phase architecture (EXPLORE/SEEK/LOCALIZE) with proven phase transition logic (from foraging theory: Viswanathan et al. 1999 Nature).

4. Comprehensive baseline suite including max-concentration reactive (reviewer-requested).

5. Conformal prediction integration (NeurIPS 2025 Fb-CP) for distribution-free uncertainty quantification of altitude estimates.