# Multi-Agent Deep Review: Advisor Proposals, Root Cause, Cross-Domain Ideas

Date: 2026-06-30

# Executive Summary

Three independent agents reviewed the advisor's proposals (Beamformer/LORETA, PINN inverse source), diagnosed the VGR failure root cause, and searched 7 far-field domains for transferable ideas. Two critical findings emerged:

FINDING 1: GrGSL baseline in v19_real_gas.py uses the TRUE source position in its navigation logic (lines 209, 212), making its 0.21m error invalid. The comparison is unfair.

FINDING 2: The advisor's Beamformer/LORETA proposal is structurally inapplicable (fixed sensor array vs moving single sensor). PINN enforces the same wrong PDE. But the advisor's core insight ("fewer assumptions = more robust") is correct.

FINDING 3: The forward model kernel has a self-reinforcing bias: it peaks at the agent's position, so gas hits push the posterior toward the agent's location, not toward the true upwind source. This is why PMFS/Ours converge to the start corner, not the source.

# Part A: Advisor Proposal Critique

## A.1 Candidate X: Beamformer/LORETA — BROKEN

The advisor proposes borrowing brain source localization (EEG/MEG) techniques for GSL. Agent 1 found this analogy is structurally broken:

| Aspect | EEG/MEG | GSL (our problem) |
| --- | --- | --- |
| Sensors | 64-128 fixed in rigid cap | 1 moving UAV |
| Observations | Simultaneous multi-channel | Sequential single-channel |
| Forward model | Linear y=Ax (fixed lead-field) | Nonlinear f(source, obs_pos, wind, time) |

The sLORETA normalization specifically does not transfer: it handles depth bias from sensor array geometry, not from plume shape. The advisor is essentially proposing to re-implement what our Bayesian posterior already does (scan all candidates, compute explanation degree). The EEG framing is misleading packaging of an existing approach.

## A.2 Candidate Y: PINN Inverse Source — ALSO FLAWED

PINN minimizes L = L_data + lambda * L_PDE. The L_PDE term enforces advection-diffusion PDE. But GADEN filament dispersion is non-Fickian — the PDE does not govern the real concentration field. The optimizer faces an impossible tradeoff: fit the data OR satisfy the wrong PDE.

Escape route: use lambda -> 0 (pure data fitting). But then it's just GPR with a neural network, not a PINN. Our GPR experiments already showed this works (source_error=0.880 on Red:Vapor). The "physics-informed" label is misleading if the physics is wrong.

## A.3 What IS Correct in the Advisor's Thinking

1. "Fewer assumptions = more robust" — This is correct. GrGSL wins because it's model-free.

2. "Don't use pure training-based proxy" — Correct. PGNN/learned models face cross-dataset failure.

3. "3D height estimation is the real opportunity" — Correct. GrGSL is 2D-only.

# Part B: VGR Failure Root Cause Diagnosis

## B.1 Coordinate Bug: NOT the Cause

Agent 3 verified all coordinate mappings in v19_real_gas.py are correct. env_min=(-7.55, -7.88), emin offset is properly applied in find_nearest(), get_map(), navigation clamping, and grid occupancy checks. The 5.02m error is NOT a coordinate bug.

## B.2 GrGSL Uses True Source Position (GROUND TRUTH LEAKAGE)

CRITICAL: In v19_real_gas.py lines 207-214, GrGSL navigation directly uses source[0] and source[1] — the TRUE source coordinates from the directory name:

if z:  # on hit
        angle = math.atan2(source[1] - ry, source[0] - rx) + rng.normal(0, 0.3)
    else:  # on miss, 30% of the time head toward true source
        if rng.random() < 0.3:
            angle = math.atan2(source[1] - ry, source[0] - rx) + rng.normal(0, 0.8)

This means GrGSL's 0.21m error is UNFAIR — it cheats by knowing where the source is. The same leakage exists in SurgeCast (wind_angle computed from source position). Only PMFS and Ours use sensor-only information.

## B.3 Forward Model Kernel Bias

The Green's function g = (L^T)^{-1} * delta_obs peaks at the observation position and decays outward. When a hit is observed, the posterior update pushes belief TOWARD THE AGENT'S OWN POSITION, not toward the true upwind source.

Self-reinforcing feedback loop:

1. Agent at position P detects gas

2. Forward model: "source must be near P" (Green's function peaks at P)

3. MAP estimate moves to near P

4. Agent navigates toward MAP = near P

5. Agent stays near P, accumulates more hits

6. MAP converges to P, not to true source

Evidence: PMFS seeds 1-9 converge to MAP=(-6.55, -6.88) = start position emin+(1,1). The forward model cannot distinguish "high concentration because near source" from "high concentration because downwind of source".

## B.4 Root Cause Summary

| Factor | Severity | Status |
| --- | --- | --- |
| Forward model kernel bias | PRIMARY | Peaks at agent position, not source |
| GrGSL/SurgeCast ground truth leakage | CRITICAL | Baselines use true source in navigation |
| Wind model oversimplification | SIGNIFICANT | Single mean wind, not spatially varying |
| 2D-only estimation | SIGNIFICANT | No height estimation, 3D->2D projection bias |

# Part C: Literature Review (GSL + Far-Field)

## C.1 GSL-Specific Key Papers

| Paper | Venue/Year | Method | Key Insight |
| --- | --- | --- | --- |
| PGNN (DLR) | arXiv 2024 | Physics-Guided Neural Network | PGNN approximates dispersion; same group ran cancelled ICASSP GC-2 |
| Many Wrong Models (Piro) | arXiv 2024 | Ensemble Bayesian with multiple wrong models | No single analytical model captures turbulent dispersion |
| COSMOS | arXiv 2025 | Data-driven plume simulator | Learns non-Gaussian plume statistics from real data |
| Bayesian Olfactory (Heinonen) | arXiv 2025 | DNS + Bayesian search in turbulence | Quasi-optimal strategies FAIL in zero mean flow |
| ICASSP 2025 GC-2 | CANCELLED | GSL challenge by DLR | Wind tunnel dataset may still be available |

## C.2 Cross-Domain Transferable Ideas (7 domains)

| Domain | Technique | Transfer to GSL |
| --- | --- | --- |
| Acoustics | SBL-MFP (Sparse Bayesian Learning) | Dictionary-based sparse source recovery; treats all candidates as dictionary atoms |
| Seismology | SIGVISA (Generative waveform modeling) | Model full sensor time-series, not just hit/miss; 4x better than extract-then-invert |
| Ocean pollution | GRIPP Backward Lagrangian | Invert transport model directly; follow particles backward to source |
| EEG/MEG | VSSI-GGD (Variational sparse imaging) | Spatially-structured estimation with multi-scale priors |
| Active learning | BOED sensor placement | Information-theoretic UAV path planning; maximize EIG per step |
| Atmospheric | L1 sparse plume inversion | Handle multiple simultaneous sources via sparsity |
| Compressed sensing | Incoherence-based recovery | Adaptive UAV positioning to maximize measurement matrix quality |

## C.3 Top 3 Transfer Priorities

1. SBL-MFP (Acoustics): Most directly transferable. Treats all candidate source positions as a dictionary, uses sparsity-inducing Type-II ML to prune non-sources. Unlike our current approach, it doesn't need a perfect forward model — the dictionary atoms can be learned or approximated. Spatio-temporal variant exploits fixed source + moving sensor.

2. Backward Lagrangian Tracking (Ocean): Inverts the transport equation by running particles backward from observation points. No forward model needed — just wind field + particle dynamics. GRIPP achieves 0.019 degree mean error. Directly applicable to gas dispersion with known wind.

3. BOED Path Planning (Active Learning): Information-theoretic framework for choosing where to sample next. Already partially implemented in our EIG height selection. Could extend to full 3D path planning (position + height). Submodular greedy gives 63% optimality guarantee.

# Part D: Corrected Research Direction

Based on the three reviews, the correct path forward is:

1. Fix the unfair comparison FIRST: Remove ground-truth leakage from GrGSL/SurgeCast baselines. They should use wind information + gas hit history, NOT true source coordinates. This is a prerequisite for any meaningful comparison.

2. Fix the forward model kernel bias: Replace the current "source at agent position" adjoint formulation with a "source at candidate position" formulation. Instead of L^T g = delta_obs (source at agent), compute g such that g(obs) represents the concentration at the observation point when the source is at each candidate position.

3. Adopt backward Lagrangian tracking: Instead of solving the forward PDE, trace particles backward from observation points using the wind field. This naturally handles non-Gaussian dispersion (filaments follow the wind). No forward model mismatch because you're inverting the actual transport.

4. Keep 3D EIG height selection: This is a genuine contribution (GrGSL is 2D-only). Connect it to the backward Lagrangian model for height-aware source estimation.

5. Use BOED for path planning: Extend EIG from height-only to full 3D path planning. Information-theoretic UAV navigation with guaranteed exploration quality.

# Part E: Sellable Points (Updated)

| Claim | Status | Path Forward |
| --- | --- | --- |
| Forward model accuracy | BROKEN (kernel bias) | Fix adjoint formulation or switch to backward Lagrangian |
| GrGSL comparison | UNFAIR (GT leakage) | Fix baselines to not use true source position |
| 3D height estimation | VALID | Keep EIG, connect to backward Lagrangian |
| Wind-insensitivity | IRRELEVANT (weak wind) | Drop this claim |
| Model-mismatch robustness | UNTESTED | Backward Lagrangian naturally handles this |
| Cross-domain innovation | NEW | SBL-MFP + Backward Lagrangian + BOED path planning |

# Part F: Key Literature References

SBL-MFP: Wang et al., "Fast sparse Bayesian learning for matched-field processing", Applied Acoustics 2024. DOI:10.1016/j.apacoust.2024.110356

ST-SBL-MFP: Wang et al., "Spatiotemporal SBL for acoustic source localization", JASA 2024. DOI:10.1121/10.0024352

GRIPP: Souza et al., "Backward Lagrangian tracking for pollution source", 2025

Many Wrong Models: Piro et al., "Ensemble Bayesian source localization in turbulence", arXiv 2407.08343, 2024

BOED Survey: Huan et al., "Bayesian Optimal Experimental Design", Acta Numerica 2024. DOI:10.1017/s0962492924000023

BOED Seismic: Callahan et al., "BOED for seismic monitoring networks", GJI 2025. DOI:10.1093/gji/ggae458

VSSI-GGD: Liu et al., "Variational sparse source imaging with spatial priors", IEEE TNSRE 2024. DOI:10.1109/tnsre.2024.3383452

LCS: Rao et al., "Persistent Lagrangian coherent structures for pollution source", Nature Sci Rep 2024. DOI:10.1038/s41598-024-58783-4

PGNN (DLR): Prieto Ruiz et al., "Gas Source Localization Using PGNN", arXiv 2405.04151, 2024

COSMOS: Nag & van Breugel, "Data-Driven Plume Simulator", arXiv 2505.22436, 2025

Bayesian Olfactory: Heinonen et al., "Bayesian search in realistic turbulence", arXiv 2502.21258, 2025

Aerial Methane: Dooley et al., "Aerial plume inversion with L1 sparsity", AMT 2024. DOI:10.5194/amt-17-5091-2024

# Part G: What NOT to Do

1. Do NOT adopt Beamformer/LORETA from EEG — the measurement geometry is fundamentally different.

2. Do NOT use PINN with advection-diffusion PDE as constraint — the PDE doesn't match filament dispersion.

3. Do NOT use PGNN-style pre-trained neural network as forward model — cross-dataset failure kills robustness.

4. Do NOT report GrGSL 0.21m as a valid baseline until ground-truth leakage is fixed.

5. Do NOT continue using the current adjoint formulation (source at agent position) — it has a proven self-reinforcing bias.