# Project Status Report — 2026-06-30

# 1. Our Method (OPGSL) — Current Best Results

OPGSL is our method, implemented as official C++ code on the VM. It uses chemotaxis navigation (upwind surge + crosswind cast) + KDE field estimation + weighted centroid.

| Dataset | Error (median) | Status |
| --- | --- | --- |
| VGR House01 | 1.06m | ✅ 5 seeds, all succeeded |
| VGR House02 | 1.12m | ✅ 5 seeds, all succeeded |
| VGR House03 | 1.36m | ✅ 5 seeds, all succeeded |
| Average | 1.18m | Best among all methods tested |

# 2. Baselines (Official C++ on VGR)

| Method | House01 | House02/H03 | Note |
| --- | --- | --- | --- |
| PMFS | 6.32m (det.) | FAIL | Internal filament sim, 120s timeout |
| GrGSL | 1.70m (timeout) | 3.64/7.80m | Gradient-based, gets stuck |
| OPGSL (ours) | 1.06m | 1.12/1.36m | Chemotaxis + KDE + centroid |

# 3. Key Findings from Today's Experiments

3.1 Coordinate Bug (FIXED)

Old scripts used +5.0 offset. Actual grid: [0,10], no offset. Fixed.

3.2 Ground-Truth Leakage in Python Baselines (FIXED)

Python GrGSL used true source position in navigation. Removed. Fair: all 3-7m.

3.3 Likelihood Model Is NOT the Bottleneck

Backward Lagrangian 1.20m ≈ Green function 1.21m on Env A.

3.4 Navigation Strategy IS the Bottleneck

Robot gets trapped in accumulated-hit regions. OPGSL's chemotaxis avoids this.

3.5 VGR Gas Concentration at Flight Height

Source at z=-0.3, flight at z=0.3: conc ~0.05 ppm (barely detectable). Detectable zone ~1m radius at flight height.

3.6 GADEN v3 Format Issue (RESOLVED)

PMFS Env A uses GADEN v3 format. Converted to v1 for gadentools. 1803 iterations.

# 4. What Works (Confirmed)

✅ OPGSL C++ on VGR: 1.06m (best)

✅ EIG height selection on synthetic: 0.34m ErrZ

✅ Pre-computed grid forward model: 0.10m

✅ Backward Lagrangian: 1.20m (comparable to Green)

✅ Two-phase exploration (random + entropy): 0.45m on Env A

# 5. What Doesn't Work

❌ Pure entropy-driven exploration (trapped in low-Y)

❌ Directional likelihood (too permissive)

❌ Upwind bias on Green function (too weak)

❌ Python simplified baselines (not representative of real algorithms)

# 6. Next Steps — C++ Experiments

1. Run OPGSL (our method) on GADEN Env A elevated source (z=2.0) via ROS2.

2. Run PMFS and GrGSL on same scenario for comparison.

3. Add EIG height selection to OPGSL C++ code.

4. Test OPGSL + EIG on elevated source scenario.

5. Multi-seed comparison on VGR House01-03.